DOCUMENTATION INFORMATION				         module: BDD

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

system	   : Apollo DN3000, HP 9000/S500, IBM RS/6000

file	   : bdd_fns.doc

unit-title:

ref.	   : Efficient Implementation of a BDD Package, Karl S. Brace. DAC'90

author(s) : Copyright (c) 1990-1996 G.L.J.M. Janssen

date	   : 17-DEC-1996

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Documentation for C functions and global variables and types in bdd_fns.[ch]

----------------------------------------------------------------------------

Useful constants:

-----------------

FALSE		/* 0 */

TRUE		/* 1 */

DONTCARE	/* 2 */

C preprocessor macros:

----------------------

FOR_EACH_CUBE(list, c)

END_FOR_EACH_CUBE

bdd_cofactor_pos(f, var)

bdd_cofactor_neg(f, var)

bdd_cofactor_pos_vec(F,i,j,var)

bdd_cofactor_neg_vec(F,i,j,var)

The Routines:

-------------

The functions in bdd_fns.c are based on the functions and global variables

of bdd.c. Therefore bdd_fns.h will cause bdd.h to be included if this was not

already explicitly done.

Typically the include files for an application of the bdd package are:

#include <stdio.h>

#include "alloc.h"

#include "bdd_fns.h"

Now follows a description of each function in bdd_fns.c:

Double bdd_count_sat_assignments (BDDPTR f, BDDPTR domain)

Returns the number of possible truth-assignments to the variables as specified

by the domain argument that make the function f true.

In other words, the number of minterms of the on-set of the function considered

to be defined on the domain.

It is assumed that the support of f is a sub-set of the domain variables.

If not, the program will exit.

The domain argument is supposed to be a BDD cube of positive variables, i.e.,

the conjunction of a number of variables; this cube is treated like a set:

BDD_1 represents the empty set, any other constant represents the universe,

i.e., it implicitly specifies all variables currently present (there are

bdd_nr_vars of them).

Returns 0 when f is BDD_VOID.

See also the Double package in double.[ch].

Double bdd_count_X_terms (BDDPTR f)

Returns the number of possible truth-assignments to the variables

with BDD rank numbers 0 through n-1 (inclusive) that make the function f

don't care, i.e., the functiuon value is BDD_X.

It is required that f is indeed expressed as a function over the BDD rank

domain { 0..bdd_nr_vars-1 }. Returns 0 when f is BDD_VOID.

See also the Double package in double.[ch].

int bdd_cube_p (BDDPTR f)

Is f a bdd cube?

A bdd cube is a bdd with the property that it denotes a product of literals,

i.e. variables or their negation, in other words there exists only a single

path in the bdd graph to the 1 terminal vertex and otherwise the paths lead

to the 0 vertex (BDD_X may not appear). The 1 function is by definition a

special case of a cube. For convenience we also define the 0 function to be

a cube,

Empty (BDD_VOID) and BDD_X bdd's are NOT considered to be a cube!

A cube determines a sub-space in the Boolean n-space.

int bdd_size_cube (BDDPTR cube)

Returns size of cube, i.e. the number of literals in it.

For an empty (BDD_VOID) or constant argument 0 is returned.

This routine is not reentrant; it uses a static global int variable.

int bdd_in_support (int var, BDDPTR f)

Is the variable with id number `var' in the support of bdd f?

If var is the id of a constant (BDD_0, BDD_1, or BDD_X) and/or f

is BDD_VOID then in_support will return 0.

Support is defined as the set of variables that the function depends on:

support (f) = { x | f(x=0) != f(x=1) }

LIST bdd_support_as_list_of_vars (BDDPTR f)

Returns list of variable id's (in increasing order of their ranks)

that constitute the support of the function f.

If f is a constant or BDD_VOID, a NULL_LIST is returned.

This result list should be freed using: free_list (list, 0).

BDDPTR bdd_support_as_cube (BDDPTR f)

Returns the support of the function f as a bdd cube of positive literals.

If f is BDD_VOID, this is returned. If f is constant, bdd_1 () is returned.

This routine uses bdd_support_as_list_of_vars.

NOTE: Uses ite operations, therefore make sure to protect f

argument; result is guaranteed to be protected.

LIST bdd_cube_as_list_of_vars (BDDPTR cube)

Returns list of (possibly negative) variable id's (in increasing order of

their ranks) that appear in the cube. The index will be negative when the

corresponding variable is to be interpreted as a negative literal.

If index 0 is used for a variable this causes a problem since -0 = 0.

Therefore better start assigning bdd id's at 1.

If cube is a constant or BDD_VOID, a NULL_LIST is returned.

This routine is not reentrant; it uses a static global LIST variable.

The routine's actions are not defined when the argument is not a bdd cube.

BDDPTR bdd_subst (BDDPTR f, int var, BDDPTR g)

Substitutes f for var in g. Also known as compose operation.

Let g = g(x1, x2, x3, ...) then bdd_subst (f, x2, g) = g(x1, f, x3, ...).

When g does not depend on var, g is returned.

If either f or g is BDD_VOID, BDD_VOID is returned.

If var is the id of a constant, g is returned.

NOTE: Uses ite operations, therefore make sure to protect f and g

arguments; result is guaranteed to be protected.

BDDPTR bdd_subst2 (BDDPTR h, BDDPTR g, BDDPTR f)

Substitutes h for g in f. The intention is that g is a subfunction of

f and that it is replaced by the function h.

If g is not a subfunction of f then f is returned.

If either of the arguments is BDD_VOID that is returned.

NOTE: Uses ite operations, therefore make sure to protect f, g and h

arguments; result is guaranteed to be protected.

BDDPTR bdd_compose (BDDPTR f, BDDPTR g, BDDPTR h)

In function f replaces g with the function h. If one or more of the arguments

is BDD_VOID this is immediately returned. Otherwise, when g is a constant

`bdd_replace_consts' is used to change the constant g in f to h. Finally,

if g is any non-constant function then returns

`bdd_subst (h, BDD_TOP_VAR (g), f)'.

NOTE: Uses ite operations, therefore make sure to protect f, g and h

arguments; result is guaranteed to be protected.

int bdd_unate_in (BDDPTR f, int var)

Is the function f unate in the variable with variable id var?

If var is the id of a constant (BDD_0, BDD_1, or BDD_X) and/or f

is BDD_VOID then unate_in will return 0. If f is constant or var is not

in the support of f 1 is returned.

A function is monotone or unate in a variable v iff either f is

monotone non-decreasing in v or f is monotone non-increasing in v.

A function is monotone non-decreasing in a variable v iff a change

of v from 0 to 1 can under no circumstances (assignment to variables

other than v) result in a change of f from 1 to 0.

A function is monotone non-increasing in a variable v iff a change

of v from 0 to 1 can under no circumstances (assignment to variables

other than v) result in a change of f from 0 to 1.

NOTE: Uses ite operations, therefore make sure to protect f argument.

int bdd_has_dontcare (BDDPTR f)

Can f take on the DONTCARE value?

BDDPTR bdd_on_set (BDDPTR f)

Interpret all X's (BDD_X) in f as being 0.

If f is a proper (fully-specified) function then on_set (f) = f.

NOTE: Uses ite operations, therefore make sure to protect f

argument; result is guaranteed to be protected.

BDDPTR bdd_off_set (BDDPTR f)

Interpret all X's in f as being 1, and then take complement.

If f is a proper (fully-specified) function then off_set (f) = f'.

NOTE: Uses ite operations, therefore make sure to protect f

argument; result is guaranteed to be protected.

BDDPTR bdd_dontcare_set (BDDPTR f)

Returns function that is 1 in precisely those cases where f would be

don't care.

If f is a proper function then dontcare_set (f) = 0.

NOTE: Uses ite operations, therefore make sure to protect f

argument; result is guaranteed to be protected.

BDDPTR bdd_replace_consts (BDDPTR f,

BDDPTR ONE_is, BDDPTR ZERO_is, BDDPTR X_is)

Auxiliary function for calculation of various sets.

In f, replace every reference to the 1 node with ONE_is,

and replace every reference to the 0 node with ZERO_is,

and replace every reference to the X node with X_is.

All `changes' or better updates should be regarded as being executed in

parallel. So doing a (0->1, 1->0) change will set an original 0 to 1 but

NOT back to 0 again.

Note that:

replace_consts (f,  BDD_1, BDD_0, BDD_X) = f

replace_consts (f,      g, BDD_0, BDD_X) = AND_BDD (f, g)

replace_consts (f, BDD_0,  BDD_1, BDD_X) = NOT_BDD (f)

NOTE: Uses ite operations, therefore make sure to protect all

arguments; result is guaranteed to be protected.

BDDPTR bdd_diff (BDDPTR f, LIST vars)

Calculates the boolean difference of f with respect to the variables in vars.

vars must be a list of BDD id's in ascending order of their ranks.

Returns f if vars is an empty list, BDD_0 if f is a constant.

Returns BDD_VOID if f is BDD_VOID.

For a single variable x, bdd_diff (f, x) is defined by (f|x xor f|x');

in case of more variables, this result generalizes to:

D x1,x2,...xn.f = D x1,x2,...,xn-1 . (D xn.f).

NOTE: Uses ite operations, therefore make sure to protect f

argument; result is guaranteed to be protected.

BDDPTR bdd_none_of_list (LIST args)

Say, args = (f1, f2, ..., fn). Calculates f1' & f2' ... & fn'.

Processes list in reverse order; this is fastest for fi's that are variable

BDDs in increasing rank order, which is quite often the case in some

application. Returns BDD_1 for empty list argument.

NOTE: Uses ite operations, therefore make sure to protect all args BDDs.

Result is guaranteed to be protected. Argument list is not

modified; should be explicitly freed by caller when no longer needed.

BDDPTR bdd_none_of_vec (BDDPTR *args, int size)

Same as bdd_none_of_list except argument is BDD vector.

BDDPTR bdd_one_of_list (LIST args)

Returns BDD expressing that precisely one of the arguments is true, i.e.,

say, args = (f1, f2, ..., fn) then calculates

f1 & none-of (rest) V f1' & one-of (rest).

Returns BDD_0 for empty argument list.

BDDPTR bdd_one_of_vec (BDDPTR *args, int size)

Same as bdd_one_of_list except argument is BDD vector.

NOTE: Uses ite operations, therefore make sure to protect all args BDDs.

Result is guaranteed to be protected. Argument list is not

modified; should be explicitly freed by caller when no longer needed.

LIST bdd_shortest_path_to_1_as_list (BDDPTR f)

Looks for satisfying truth-assignment for function f.

If one exists, will return a list of bdd nodes that constitute the

shortest satisfying path in the bdd graph for f.

BDD_X vertices in the bdd graph for f are NOT interpreted.

The root node of f is always the first node on the path and thus

the first element of the list; the terminating constant node (BDD_0 or BDD_1)

is the last node in the list.

(NOTE: bdd pointers in the list directly refer to nodes of f.)

If no satisfying truth-assignment exists, a NULL_LIST is returned.

This routine is not reentrant; it uses a static global LIST variable.

BDDPTR bdd_shortest_path_to_1_as_cube (BDDPTR f)

Looks for satisfying truth-assignment for function f.

If one exists, will return the bdd nodes that constitute the

shortest satisfying path in the bdd graph for f as a cube.

BDD_X vertices in the bdd graph for f are NOT interpreted.

The root node of f is always the first node on the path and thus

the first element of the cube. If f = 1 then 1 is returned.

If no satisfying truth-assignment exists, BDD_0 is returned.

This routine is not reentrant; it uses a static global LIST variable.

NOTE: Uses ite operations, therefore make sure to protect f

argument; result is guaranteed to be protected.

LIST bdd_sum_of_cubes_as_list (BDDPTR f)

Converts the bdd representation of f to a sum-of-products representation,

which is returned as a list of bdd cubes. The result consists of cubes that

are prime but not-necessarily irredundant. Any X's in f are interpreted

as 0, thus effectively computing the sum-of-products for the on_set of f.

If f is BDD_VOID the NULL_LIST is returned.

The bdd's in the list are all GC_PROTECT-ed. The list must thus be deleted

with "free_list (list, free_bdd)".

NOTE: Uses ite operations, therefore make sure to protect f

argument; result's list elements are guaranteed to be protected.

LIST bdd_irredundant_sum_of_cubes_as_list (BDDPTR f)

Converts the bdd representation of f to a sum-of-products representation,

which is returned as a list of bdd cubes. The result is an irredundant sum

of prime implicants expression for f. Any X's in f are interpreted

as 0, thus effectively computing the sum-of-products for the on_set of f.

If f is BDD_VOID the NULL_LIST is returned.

The bdd's in the list are all GC_PROTECT-ed. The list must thus be deleted

with "free_list (list, free_bdd)".

NOTE: Uses ite operations, therefore make sure to protect f

argument; result's list elements are guaranteed to be protected.

int bdd_traverse_cube (BDDPTR cube, void (*action) (int, int))

Assumes cube is indeed a bdd cube and then the returned value is 1.

If not, the actions are not defined, but it is guaranteed not to cause a crash

or endless loop, and 0 is returned.

For each variable in the cube (in the order of the overall variable

ordering), the routine `action (var, neg)' is called, with

var the variable id and neg is non-zero when the variable appears

negated in the cube.

If cube is a constant or BDD_VOID, no action is performed.

BDDPTR bdd_prime_implicant (BDDPTR f)

Computes a relatively small (in the sense of number of literals) prime

implicant for f. Returns this as a BDD cube. Immediately returns BDD_VOID

for BDD_VOID argument. This may be used as an alternative to

bdd_shortest_path_to_1_as_cube. The latter finds the shortest path in the BDD

but this not necessarily corresponds with the smallest prime implicant.

BDDPTR bdd_eval (BDDPTR f, BDDPTR cube)

f is evaluated for values of its supporting variables as determined by the

cube argument. The rules are:

v := 0 when cube = 0 or v appears negative in the cube,

or doesn't appear at all;

v := 1 when cube = 1 or v appears positive in the cube,

The (necessary) constant value f evaluates to, is returned as a bdd.

If f is a constant or BDD_VOID, returns f, else

if cube is BDD_VOID or BDD_X, returns cube.

When cube is not a proper bdd cube, the actions are not defined, the

result is some constant,

but it is guaranteed not to cause a crash or endless loop.

Examples: bdd_eval (a b + c', a c) = 0  [a=1,b=0,c=1]

bdd_eval (a b + c', c')  = 1  [a=0,b=0,c=0]

bdd_eval (a b + c', d)   = 1  [a=0,b=0,c=0]

bdd_eval (a b + c', 0)   = 1  [a=0,b=0,c=0]

bdd_eval (a b + c', 1)   = 1  [a=1,b=1,c=1]

BDDPTR bdd_invert_input (BDDPTR f, int var)

Returns function g such that g(x1, var', ...) = f(x1, var, ...).

That is g is the function f with the variable var substituted by its

negation. If f is a constant or BDD_VOID, f is returned.

NOTE: Uses ite operations, therefore make sure to protect f

argument; result is guaranteed to be protected.

BDDPTR bdd_invert_inputs (BDDPTR f)

Returns function g such that g(x1', x2', ...) = f(x1, x2, ...).

That is, g is the function f with all variables substituted by their

negation. If f is a constant or BDD_VOID, f is returned.

NOTE: Uses ite operations, therefore make sure to protect f

argument; result is guaranteed to be protected.

int bdd_depth (BDDPTR f)

Returns the depth of BDD f. The depth is defined as the length of the longest

path starting at the root node, in fact it is the number of nodes (constant

nodes exclusive) on that path. The depth of a constant or BDD_VOID is 0.

void bdd_use_sop_cache_switch (int on)

Determining a sum-of-cubes representation for a BDD f with

bdd_sum_of_cubes_as_list (f) can be greatly speeded up when a cache is used

for intermediate results. By default, the routine indeed uses such a cache

and this cache will remain valid throughout the program unless

bdd_cleanup_sop_cache () is explicitly called which will clear the cache

entries.

bdd_use_sop_cache_switch (on) allows control over the use of this cache: with

a zero argument value use of the cache is inhibited and also the cache entry

are cleaned up (implicit call to bdd_cleanup_sop_cache). A non-zero argument

turns use of the cache on.

void bdd_cleanup_sop_cache (void)

Clears the cache that is used in holding intermediate results while

determining the sum-of-cubes representation of a BDD. The use of the cache

is not switched off however.

Note: bdd_quit calls bdd_cleanup_sop_cache.

BDDPTR bdd_cube_factor (BDDPTR f)

Determines the maximum `core' cube in f, i.e., a cube with a

maximum number of literals such that f -> c is a tautology.

Structurally this means that every satisfying path in the BDD for

f will pass through the core cube nodes.

Thus the core cube may be interpreted as expressing the bit-wise xnor

over all minterms (over the support variables) of f.

In a truth-table this is found by listing all patterns that make f true

and collecting those variables whose columns contain either all 0's or

all 1's. In other words, assigning false to a literal in the core cube

for sure implies that f will evaluate to false no matter how the other

variables will be assigned.

For a non-0 f the fact that the core cube consists of no literals

will be represented by BDD_1.

For the special case f = 0, we define the result to be BDD_0.

(BDD_X nodes will be interpreted as BDD_1).

Let c = bdd_cube_factor (f), then f may be written as f = c & g.

g can be obtained either by cofactoring:

g = bdd_constrain (f, c);

or using existential quantification:

LIST vars;

vars = bdd_support_as_list_of_vars (c);

g = bdd_quantify (1, f, vars);

free_list (vars, 0);