Chips and Technologies Super VGA Chip Sets:

82c451  256k DRAM  max 800x600 16col

82c452    1M DRAM  max 640x480 256col, 1024x768 16col

82c453    1M VRAM  max 800x600 256 col

82c455  256k DRAM  Flat Panel version

82c456  256k DRAM  do

94h (R/W):  Setup Control Register for Microchannel boards

bit 0-2  Reserved

3  Enables Adapter VGA if set

4  Enters Setup Mode if set

5-7  Reserved

Note: This is the same register as 46E8h.

100h (R): Microchannel ID low

bit 0-7  Bit 0-7 of Microchannel Card ID

101h (R): Microchannel ID high

bit 0-7  Bit 8-15 of Microchannel Card ID

102h (R/W): Global Enable

bit 0  VGA Awake/Asleep

103h (R/W): Multible Enable             (Only in Setup Mode)

bit 0-4  Multible VGA Enable

6  Extension registers at 3B6h/7h if set,

3D6h/7h if not.

7  Enable VGA extensions if set

104h (R): Global ID (Setup)             (Only in Setup Mode)

bit 0-7  Chip I/D.  0A5h if Chips and Tech Chip set.

3C3h (R/W): Setup Control PS/2

bit 0  Enables motherboard VGA if set

4  Enters Setup mode if set

3d4h index 22h (R/W): CPU Data Latch or Color Compare from last read

3d4h index 24h (R/W): Attribute Controller flip/flop

3d6h index  0  (R): Chip Version

bit 0-3  Revision number

4-7  Chipcode:

0: 451  1:452  2:455  3:453  5:456

3d6h index  1  (R/W): DIP Switch

3d6h index  2  (R/W): CPU Interface

bit 0  16bit memory if set

1  16 bit I/O if set

(82c453 Only) Fast Font Enable   ???

The byte written to memory is used as a mask

for painting foreground color to the pixels

with the corresponding bit set and background

color to the rest.

2  Fast MCA buscycle decoding if set

3-4  (82c453 Only) Attribute port pairing

0: Normal Attribute adressing

1: 3C1h is both read and write, 8 and 16 bit.

2: 3C1h is both read and write, 8 bit only.

5  (82c453 Only) 10 bit I/O decodning if set, 16 bit else

6  (82c453 Only) Pel Panning Control

7  (Read Only) Attribute flip-flop status

3d6h index  3  (R/W): ROM Interface

bit 0  Disable on-card ROM if set

3d6h index  4  (R/W): Memory Mapping

bit 2  (82c451/5/6) Enable bank access if set

0-1  (82c452/3) Display Memory Size:

0: 256Kb, 1: 512Kb, 2: 1Mb.

2  (82c452/3) CRTC Address can cross bank boundaries if set

3d6h index  5h (R/W): Sequencer Control

3d6h index  6h (R/W): DRAM Interface

3d6h index  8h (R/W): General Purpose

3d6h index  9h (R/W): General Purpose

3d6h index  Ah (R/W): Cursor Address Top            (82c452/3 Only)

bit 0-1  Cursor Address bit 16,17

2-7  Reserved

3d6h index  Bh (R/W): CPU Paging                    (82c451/5/6)

bit 0-1  Bank number in 64k chunks.

Note: This Bank register is used if in a 256 color mode and

the chip is a 82c451/5/6.

3d6h index  Bh (R/W): Memory Paging Register        (82c452/3)

0  Enable extended paging (256 color paging) if set

1  Dual Page Enable if set.

2  CPU Address divide by 4 (256 color addressing)

3d6h index  Ch (R/W): Start Address Top             (82c452/3 Only)

bit 0-1  Display Start Address bit 16,17.

3d6h index  Dh (R/W): Auxiliary Offset Register

bit 0  Bit 8 of Offset field. If set each line is >255 words.

1  Bit 8 of simulated Offset field.

3d6h index  Eh (R/W): Text Mode                     (82c452 Only)

bit 0  Extended text Mode Control ??

1  Enable anti-aliased fonts if set

3d6h index 10h (R/W): Single/Low Map                (82c452/3 Only)

bit 0-5  (82c452) Bank no in 4K/16K chunks.

0-7  (82c453) Bank no in 1K/4K chunks.

Note: This Bank register is used if in single-paging mode

or if addressing the lower half (32 or 64Kb) of the

adapters address range.

3d6h index 11h (R/W): High Map                      (82c452/3 Only)

bit 0-5  (82c452) Bank no in 4K/16K chunks.

0-7  (82c453) Bank no in 1K/4K chunks.

Note: This Bank register is used if addressing the upper

half (32 or 64Kb) of the adapters address range.

3d6h index 20h (R/W): Sliding Unit Delay

3d6h index 21h (R/W): Sliding Hold A

3d6h index 22h (R/W): Sliding Hold B

3d6h index 23h (R/W): Write Mask Control            (82c453 Only)

bit 0  Enable VRAM Write Mask function if set

1-2  Write Bit Mask Select:

0: Write Bit Mask Pattern Register (3d6h index 24h)

1: Graphics Controller Bit Mask    (3CEh index 8)

2: Rotated CPU byte

3  Enable Fast Read/Modify/Write function if set

3d6h index 24h (R/W): Write Bit Mask Pattern        (82c453 Only)

bit 0-7  Write Bit Mask (if 3d6h index 23h bit 1-2 =0)

3d6h index 26h (R/W): Configuration   (82c453 Only)

bit 0  PC/AT if set, PS/2 else

1-2  VRAM memory

0: 512k   16 chips of  64k x4

1: 512k    4 chips of 256k x4

2:   1M    8 chips of 256k x4

3: 512k    8 chips of  64k x4  ?????

maybe 256k ??

3d6h index 27h (R/W): Force Sync State

3d6h index 28h (R/W): Video Interface

3d6h index 29h (R/W): External Sync Control

3d6h index 2Ah (R/W): Frame Interrupt Count         (82c452 Only)

bit 0-4  Generate Vertical Interrupt every (n+1) frames

3d6h index 2Bh (R/W): Default Video

3d6h index 2Ch (R/W): Force Horizontal High

3d6h index 2Dh (R/W): Force Horizontal Low

3d6h index 2Eh (R/W): Force Vertical High

3d6h index 2Fh (R/W): Force Vertical Low

3d6h index 30h (R/W): Graphics Cursor Start Address High

bit 0-7  Bit 8-15 of the Cursor Start Address.

3d6h index 31h (R/W): Graphics Cursor Start Address Low

bit 0-7  Lowest 8 bits of the Cursor Start address.

3d6h index 30h and index Ah forms the upper 10 bits.

In 256 color modes this address has a granularity

of 16 bytes and 4 bytes in 16 color modes.

3d6h index 32h (R/W): Graphics Cursor End Address

bit 0-7  End address of the cursor bit map.

3d6h index 33h (R/W): Graphics Cursor X Position High

bit 0-3  Bits 8-11 of the X coordinate of the cursor.

3d6h index 34h (R/W): Graphics Cursor X Position Low

bit 0-7  Lower 8 bits of the X coordinate of the cursor.

3d6h index 35h (R/W): Graphics Cursor Y Position High

bit 0-3  Bits 8-11 of the Y coordinate of the cursor.

3d6h index 36h (R/W): Graphics Cursor Y Position Low

bit 0-7  Lower 8 bits of the cursor Y coordinate.

3d6h index 37h (R/W): Graphics Cursor Mode

bit 0  Cursor Enabled if set

1  Cursor Status enable

2  Horizontal Zoom. Zoom to 64 pixels wide if set.

(Normally 32 pixels wide).

3  Cursor Blink enabled if set

4  Cursor Blink Rate. 8 frames if clear, 16 if set

3d6h index 38h (R/W): Graphics Cursor Plane Mask

bit 0  Enables graphics cursor in bit plane 0 if set

1  Enables graphics cursor in bit plane 1 if set

2  Enables graphics cursor in bit plane 2 if set

3  Enables graphics cursor in bit plane 3 if set

3d6h index 39h (R/W): Graphics Cursor Color 0

bit 0-7  Background color of Graphics Cursor.

3d6h index 3Ah (R/W): Graphics Cursor Color 1

bit 0-7  Foreground color of Graphics Cursor.

3d6h index 44h (R/W): Scratch #0 Register           (82c453 Only)

bit 0-7  Available

3d6h index 45h (R/W): Scratch #1/Foreground Color   (82c453 Only)

bit 0-7  Used as foreground color if in Fast Font Paint mode,

Available as scratch else.

3d6h index 50h (R/W): Panel Format                  (82c455/6 Only)

bit 0-1 Frame Rate Control

0: No gray scale simulated for mono,

8 colors simulated for color panels.

1: 4 simulated colors for color panels only

(64 colors displayed).

2: 64 gray levels simulated for mono. panels only.

2-3 Pulse Width Modulation

0: No gray scales for mono or color systems.

1: 4 colors supported by the color panels only

(64 colors displayed).

2: 16 gray levels supported by the mono panels only.

3: 256 gray levels supported by the

color single panels only.

3d6h index 51h (R/W): Panel Type                    (82c455/6 Only)

bit 0  Double drive if set, single else

1  Double panel if set, single else

2-3  Type of display

0=LCD, 1=CRT, 2=Plasma or Electrolum.

4-5  0=Color panel 3 bit data pack

1=Color Panel 1 bit data pack

2=Monochrome Panel

6  Flat Panel Compatability enabled if set

7  Text Video output polarity

3d6h index 52h (R/W): Panel Size                    (82c455/6 Only)

bit 0-1  Horizontal Size of panel

1=640 pixels, 2=720 pixels

3-6  Vertical Size of panel

1=200 lines, 2=350, 4=400, 8=480 lines

3d6h index 55h (R/W): Text Mode 350_A               (82c455/6 Only)

bit 0-3  (Number of blank lines)-1 inserted between text rows

I.e.  if 5, insert 6 blank lines after a text line.

4  If clear lines are inserted.

Note: This register is used if in a 350 line text mode

and fonts are larger than 8 lines.

3d6h index 56h (R/W): Text Mode 350_B               (82c455/6 Only)

bit 0-3  (Number of blank lines)-1 inserted between text rows

4  If clear lines are inserted.

Note: This register is used if in a 350 line text mode

and fonts are smaller than or equal to 8 lines.

3d6h index 57h (R/W): Text Mode 400                 (82c455/6 Only)

bit 0-3  (Number of blank lines)-1 inserted between text rows

4  If clear lines are inserted.

Note: This register is used if in a 400 line text mode.

3d6h index 58h (R/W): Graphics Mode 350             (82c455/6 Only)

bit 0-3  Number of scan lines between stretch/delete

4  Enable vertical Stretching if set

5  Enable vertical deletion if set

6  Increment (0-3) every other period ??

Note: This register is used if in a 350 line graphics mode.

3d6h index 59h (R/W): Graphics Mode 400             (82c455/6 Only)

bit 0-3  Number of scan lines between stretch/delete

4  Enable vertical Stretching if set

5  Enable vertical deletion if set

6  Increment (0-3) every other period ??

Note: This register is used if in a 400 line graphics mode.

3d6h index 60h (R/W): Blink Rate Control            (82c455/6 Only)

bit 0-5  Blink Rate.

Character Blink Freq = Vertical sync Freq * (Blink rate+1)

Cursor blink freq = Character Blink Freq *2.

6-7  Blink Cycle  1=25%, 2=50%, 3=75%

3d6h index 61h (R/W): Smartmap Control              (82c455/6 Only)

bit 0  Enable Smartmap if set

1-4  Threshold for (Foreground - Background) diff

if diff less than the threshold the foreground and

background colors will be spread (See 3d6h index 62h).

5  Smartmap Saturation value

6  (82c456 Only) Enhanced text if set

(reverses attributes 7h and Fh)

3d6h index 62h (R/W): Smartmap Shift Parameter      (82c455/6 Only)

bit 0-3  Number of levels to shift foreground color

when too little difference (See 3d6h index 61h bit 1-4).

4-7  No of levels to shift background color.

3d6h index 63h (R/W): Graphics Color Mapping Control (82c455/6 Only)

bit 0-3  Threshold color value for mono output.

All colors >= this value will be set to 1,

all lower to 0.

4  Use upper 4 bits of 256 color if set, lower if not.

5  Enable internal color lookup table if set

6  Write protect internal color look up table

7  Graphics output polarity

3d6h index 6Dh (R/W): FRC and Palette Control       (82c456 Only)

bit 3  Enable Frame Rate Control

4-5  Maximum number of gray levels.

6-7  Usage of External Palette:

0: Bypass

1: Bypass for 16 color modes, use for 256 color.

2: Use

3: 16 grays for 16 color modes, 64 for 256 color.

3d6h index 6Eh (R/W): Polynomial FRC Control        (82c456 Only)

bit 0-3  Polynomial N value for Frame Rate Control

4-7  Polynomial M value.

46E8h (R/W): Setup Control PC/AT Register

bit 0-2  Reserved

3  Enables Adapter VGA if set

4  Enters Setup Mode if set

5-7  Reserved

Note: This is the same register as 94h.

Most every index of 3d6h is used by one one or more chip.

Bank Switching:

Bank switching is dependent on Chip version:

16 color modes       256 color modes

Chip         #bank regs   #Banks Granularity   #banks Granularity

82c451/5/6      1                                  4    64Kbytes

82c452          2            64     4Kbytes       64    16Kbytes

82c453          2           256     1Kbytes      256     4Kbytes

For the 82c452 & 3  the window to display memory can start on

any boundry fitting the granularity of the chip/display mode.

When using 2 bankregisters, the adress range available to the

adapter is split equally between the two bank registers. I.e.

A000h-A7FFh uses one bank, and A800h-AFFFh the other.

(Or A000h-AFFFh and B000h-BFFFh respectively if using the full

128 Kbytes range).

ID Chips and Technologes Chip Set:

{Enter Setup Mode}

port[$3c3]:=port[$3c3] or 16;

if port[$104]=$a5  then CHIPS_TECH_CHIP_SET !!!!!

{Leave Setup Mode}

port[$3c3]:=port[$3c3] and $ef;

ID Which Chips and Tech VGA

{Extension Enabled}

port[$3d6]:=0;

case port[$3d7] shr 4 of

2:CHIP_82c455 !!!

3:CHIP_82c453 !!!

5:CHIP_82c456 !!!

1:begin

port[$3d6]:=$3a;

old_value:=port[$3d7];

port[$3d7]:=$aa;

value:=port[$3d7];

port[$3d7]:=old_value;

if value=$aa then CHIP_82c452 !!!

else CHIP_82x451 !!!

end;

end;

Video Modes:

60h T  132   25  16  (8x16)

61h T  132   50  16  (8x8)

6Ah G  800  600  16  planar

70h G  800  600  16  planar

71h G  960  720  16  planar   Cardinal only!

72h G 1024  768  16  planar

78h G  640  400 256  packed   Not documented/not all boards

79h G  640  480 256  packed

7Ah G  720  540 256  packed   Not documented/not all boards

7Bh G  800  600 256  packed

7Ch G  800  600 256  packed   (82c453 Only)

7Eh G 1024  768 256  packed   (82c453 Only)

Bios Extensions:

----------105F00-----------------------------

INT 10 - Get Controller Information       (Chips and Technologies Super VGA)

AX = 5F00h

Return: AL = 5F  If extended VGA control function supported

BL = CHIP Type:

Bits 4-7:

0=82c451

1=82c452

2=82c455

3=82c453

5=82c456

Bits 0-3:  Revision Number

BH = Video Memory Size

0=256 Kbytes

1=512 Kbytes

2=1 Megabyte

CX = Miscellaneous Information

Bit 0  Dac Size. 0=6bit, 1=8bit

1  System Environment. 0=PC/AT, 1=PS/2

2  Extended text modes supported by BIOS

3  Reserved

4  Extended graphics modes supported by BIOS

5  Resserved

6  Graphics Cursor supported by BIOS

7  Anti Alias font supported by BIOS

8  Preprogrammed emulation supported by BIOS

9  Auto emulation supported by BIOS

10  Variable mode set at cold boot supported by BIOS

11  Variable mode set at warm boot supported by BIOS

12  Emulation mode set at cold boot supported by BIOS

13  Emulation mode set at warm boot supported by BIOS

14-15  Reserved

----------105F01-----------------------------

INT 10 - Set Emulation Mode               (Chips and Technologies Super VGA)

AX = 5F01h

BL = Operation Mode

0-1 Reserved

2 Enable CGA Emulation

3 Enable MDA Emulation

4 Enable Hercules Emulation

5 Enable EGA Emulation

6 Enable VGA Emulation

Return: AL = 5Fh  If function supported

AH = Return Status

1 If Function Succesfull, 0 else

----------105F02-----------------------------

INT 10 - Auto Emulation Control           (Chips and Technologies Super VGA)

AX = 5F02h  Auto Emulation Control

BL = Selection

0= Enable Auto Emulation

1= Disable Auto Emulation

Return: AL = 5Fh  If function supported

AH = Return Status

1 If Function Succesfull, 0 else

----------105F03-----------------------------

INT 10 - Set Power-on Video Configuration (Chips and Technologies Super VGA)

AX = 5F03h

BL = Configuration

0:  Set display mode as specified in the CX register

at power-up.

CL=Display Mode

CH=Bits 0-1 Scanlines

0=200 Lines

1=350 Lines

2=400 Lines

Bit    7 Performance

0= Reset after next boot

1= Set until changed

1:  Set Emulation mode as specified in the CX register

at power-up.

CL=Emulation Mode (See 5F01h)

CH=Bit 7 Performance

0= Reset after next boot

1= Set until changed

Return: AL = 5Fh  If function supported

AH = Return Status

1 If Function Succesfull, 0 else

----------105F90-----------------------------

INT 10 - Return Save/Restore buffer size  (Chips and Technologies Super VGA)

AX = 5F90h

CX = Mask State

Bit 0  Save/Restore video hardware

1  Save/Restore BIOS data state

2  Save/Restore DAC state

15  Save/Restore type

0= Save/Restore All state information

1= Save/Restore super state information

Return: AL = 5Fh  If function supported

BX = Number of 64byte blocks required

----------105F91-----------------------------

INT 10 - Save State                       (Chips and Technologies Super VGA)

AX = 5F91h

CX = Mask State

Bit 0  Save video hardware

1  Save BIOS data state

2  Save DAC state

15  Save type

0= Save All state information

1= Save super state information

ES:BX -> Buffer to save in.

Return: AL = 5Fh  If function supported

----------105F92-----------------------------

INT 10 - Restore State                    (Chips and Technologies Super VGA)

AX = 5F92h

CX = Mask State

Bit 0  Restore video hardware

1  Restore BIOS data state

2  Restore DAC state

15  Restore type

0= Restore All state information

1= Restore super state information

ES:BX -> Buffer to restore from.

Return: AL = 5Fh  If function supported