GENOA SuperVGA Chips.

Genoa 5100/5200  Tseng ET3000 chips 256 Kbytes

5300/5400  Tseng ET3000 chips 512 Kbytes

6100       Own chip 256 Kbytes ??

6200/6300  Own chip 256 Kbytes   max  800x600/16 or 640x400/256

6400/6600  Own chip 512 Kbytes   max 1024x768/16 or 800x600/256

6600 is the 6400 for MicroChannel

64/6600A support 70Hz refresh rate.

3C3h (R/W): Global Enable PS/2 Register

bit 0  VGA Microchannel enabled if set

3C4h index  5  (R/W): Configuration Register

bit 0  (R) PC/AT if set, PS/2 else

1  (R) 8bit bus if set, 16bit else

2  (R) 8bit ROM if set, 16bit else

3  Reserved

4  (R) I/O address 3xx if set, 2xx else

5-6  ROM BIOS size:

0: 24kb ROM

1: 30kb ROM

2: 32kb ROM

3:  0kb ROM

7  8 simultaneous fonts if set, 2 else

3C4h index  6  (R/W): Memory Segment Register

bit 0-2  Read bank no.

3-5  Write bank no.

6  If clear use 3C2h bit 5 as A16 rather than

bit 0/3 of this register.

3C4h index  7  (R/W): Enhanced Control #2

bit 0  External Clock select (bit 2 of a 3bit field).

1  Allow frequencies above 50MHz

2  16 bit video memory access if set

3  VGA on Motherboard if set, Card else

4  Reserved

5  TTL monitor if set, Analog else

6  Non-Maskable Interrupts Enabled if set

7  Reserved

3C4h index  8  (R/W): Enhanced Control #3

bit 0  MDA,Hercules or CGA emulation if set

1  MonoChrome Autoswitch if set

Causes NMI if 3B8h is written to

2  Color Autoswitch if set

Causes NMI if 3D8h is written to

3  EGA operation if set, VGA else

4  60Hz Vertical if set, 70Hz else

5  Enable flicker free function

6  Enable extended memory addressing

7  Enable 1024x768 addressing

3C4h index 10h (R/W): Enhanced Control #4

bit 0  Fast Scroll if set

1  Fast Decode if set

2  Enables 2 bank access if set

3  Pre_Wait enabled if set

4-5  Reserved

6  Fast Write enabled if set

7  Select memory bank 0 or 1

3CEh index  9  (R/W): Enhanced Control #5

bit 6  If set Access even pixels in plane 0/1

and odd pixels in plane 2/3.   (Mode 7Fh)

7  If set Display even pixels from plane 0/1

and odd pixels from plane 2/3. (Mode 7Fh)

3CEh index  Ah (R/W): Program Status Register 1

bit 0-7  Scratch

3CEh index  Bh (R/W): Program Status Register 2

bit 0-7  Scratch

3d4h index 2Eh (R/W): Herchi Register

bit 0  If set Maximum Scan Line Register (3d4h index 9)

is programmable in CGA modes

1  Enables Chinese applications in Hercules mode if set

2-7  Reserved

3d4h index 2Fh (R/W): Interlace Control Register

bit 0  Interlace active if set

1  Enable Quadword addressing mode

2  Select Character Clock as memory addressing counter clock

3  Interlaced Graphics/Alpha mode

4-7  Reserved

46E8h (R): Global Enable PC/AT Register

bit 3  Adapter VGA enabled if set

Bank select:

Seperate read and write banks are selected through 3C4 index 6.

Memory locations:

$C000:$37  2 bytes   start of info table from start of ROM

Usually $C000:$B4

$C000:x    1 byte    $77

$C000:x+1  1 byte    Version.

$00 Genoa 6200 or 6300

$11 Genoa 6400 or 6600

$22 Genoa 6100

$33 Genoa 5100 or 5200   (Tseng 3000)

$55 Genoa 5300 or 5400   (Tseng 3000)

$C000:x+2  2 bytes   $6699

ID Genoa Chip Set:

if (meml[$c000:mem[$c000:$37]] and $ffff00ff)=$66990077 then

case mem[$c000:mem[$c000:$37]+1] of

$33,$55:TSENG 3000 chips with special modes;

else GENOA_VGA !!!

ID Genoa Chip Revision:

Case mem[$c000:mem[$c000:$37]+1] of

$22:GENOA_6100

0:GENOA_6200 or GENOA_6300

$11:GENOA_6400 or GENOA_6600

end;

Video Modes:

The Genoa 5xxx series boards use Tseng modes.

The 6xxx series boards use the following modes:

43h  T    80   29    2

44h  T    80   32    2

45h  T    80   44    2

46h  T   132   25    2

47h  T   132   29    2

48h  T   132   32    2

49h  T   132   44    2

58h  T    80   32   16

59h  G   720  512   16

5Ah  T   100   42?  16

5Bh  G   640  350  256 packed

5Ch  G   640  480  256 packed

5Dh  G   720  512  256 packed

5Eh  G   800  600  256 packed

5Fh  G  1024  768   16 planar

60h  T   132   25   16

61h  T   132   29   16

62h  T   132   32   16

63h  T   132   44   16

64h  T   132   60   16

6Ah  G   800  600   16 planar

6Bh  T   100   75   16

6Ch  G   800  600  256 packed

72h  T    80   60   16

73h  G   640  480   16 planar

74h  T    80   66   16

78h  T   100   75   16

79h  G   800  600   16 planar

7Dh  G   512  512  256 packed

7Eh  G   640  400  256 packed

7Fh  G  1024  768    4 planar

Mode 7Fh: 1024x768 4 color

2 bits per pixel planar mode

Even pixels are in planes 0 and 1, odd pixels are in planes 2 and 3.