OAK Technologies OTI-067 and 037C.

OTI037C apparently has max 256Kb and no banking registers.

OTI-067 has 256K or 512K and banking registers.

3DEh index  Dh (R/W):

bit 7  512K if set, 256 else.

3DEh index 11h (R/W):

bit 0-3  Bank no    (Read/Write or two separate banks ??)

4-7  Bank no

Note: Some sources gives this register as 3DFh with no index.

Bank Switching:      (OAK OTI-067 Only)

Two banks exist in reg 3DEh index 11h.

ID Oak VGA:

if exist($3DE,$d) then

if exist($3DE,$11) then OAK_037

else OAK_067

Modes:

(OTI 067)

4Fh  T   132   60  16 (8x8)

50h  T   132   25  16 (8x14)

51h  T   132   43  16 (8x8)

52h  G   800  600  16 planar

53h  G   640  480 256 packed

54h  G   800  600 256 packed

55h  G  1024  768   4 ??

56h  G  1024  768  16 planar

57h  G   768 1024   4 ??

(OTI037C "UNIQUE" VGA)

50h  T   132   25  16 (8x14)

51h  T   132   43  16 (8x8)

52h  G   800  600  16 planar

(OTI037C with NEL Electronics BIOS)

50h  G   640  480  16 planar

51h  T    80   30     (16x9)

52h  G  1024  480  16 planar

53h  T    80   25     (16x8)

54h  T   132   43     (8x8)

55h  T   132   25     (14x8)

56h  T   132   43     (8x9)

57h  T   132   25     (14x9)

58h  T    80   43     (8x8)

59h  T    80   43     (8x9)

5Ah  T    80   60     (8x8)

5Bh  G   800  600  16

5Ch  T   100   37     (16x8)

5Dh  T   100   75     (8x8)

5Eh  G   800  600  16

6Ah  G   800  600  16

6Bh  T   100   37     (16x8)